Setting up the Raspberry Pi for use in the Multi-Camera Tally Light Project

Originally from: https://td0g.ca/2020/05/27/simple-tally-light-for-obs-studio

These modified instructions created by SiliconKnight42 in Jun20, based on Raspberry Pi Zero W.  
Instructions updated again on 5 Nov 2025 to convert from old Pi OS (Raspbian bullseye and Python 3.7) and “obswebsocket” Python library to newer Pi OS (Raspberry Pi OS Trixie with Python 3.13.5) and “obsws_python” and to use the OBS Websockets v5 API.

- Burn the latest version of the Raspberry Pi OS (formerly Raspbian) Raspbian “Lite” to a microSD card.  These Tally Light devices are built on a Raspberry Pi Zero W (the original v1).
    (scroll down to View all download options)

  - I used Balena Etcher to do this, but RPi now has a Raspberry Pi Imager that might be easier.

  - If using the Raspberry Pi Imager:

    - For “Raspberry Pi Device” choose “Raspberry Pi Zero” (not Zero 2).

    - For “Operating System” choose “Raspberry Pi OS (other)” then “Raspberry Pi OS List (32-bit)”

    - For “Storage” choose the SD card writer.

- Insert the microSD card into the Raspberry Pi and power it up.

- Login (default uid/pid:  pi / raspberry) Change your password!

- If not created during the Raspberry Pi Imager config:

    - Using Network Manager (NOT raspi-config), add a new Wifi network:

    - sudo nmtui

  - Using the Raspi-config app:

    - Login as superuser: sudo su

    - Update the Raspi-config tool (using the menu item at the top level)

    - Set the Locale and Keyboard layout to US (under Localisation Options).

    - If remote access is desired, enable SSH (sudo raspi-config)

      - Select “Interface Options” then “SSH” and enable.

    - Set to autologin on boot: System Options, Boot Options, Desktop/CLI, Console Autologin, Finish

- Reboot and confirm network connectivity. (sudo reboot)

- Update the system: sudo apt update and sudo apt upgrade

  - Note: The newer “apt” command has replaced the older “apt-get” command.

- Reboot and login. (sudo reboot)

- Ensure Python 3 and pip are installed and at the latest version: 
     sudo apt install python3 python3-pip

- Setup the Virtual Environment (this is now required for Python to use PIP):

  - Create the VENV – this takes a few minutes, so be patient:     python3 -m venv .venv

  - Activate the VENV:     source .venv/bin/activate

  - Install ping modules into the VENV: pip3 install pythonping multiping
(sudo isn’t needed since we’re in the VENV)

  - Install the following Python 3 modules: pip3 install websocket-client obsws-python RPi.GPIO

- Create an empty text file to store the last OBS Websocket address:  
            nano  /home/pi/.venv/obsAddr.log

  - This existence of this file is critical – additionally you may also enter the OBS Studio PC’s IP address into the text file.  It will speed up the search, but isn’t necessary.

- Set ownership and permissions on that text file and copy it to the root: 
     chown pi /home/pi/.venv/obsAddr.log 
     chmod 777 /home/pi/.venv/obsAddr.log 
     cp /home/pi/.venv/obsAddr.log /home/pi

- Create the Python script:

  - Option 1: Use nano: sudo nano /home/pi/.venv/tallylight-5Nov25.py

    - Copy the code from my repository.  Save with Ctl-X Y.

      - Edit the cam_num_str_base variable if you don’t want to use the ‘cam ’# (e.g. cam 1, cam 2, etc.) string as a trigger in OBS for responding to the scene changes.

      - Make sure the password matches what was set in OBS Studio for the WebSockets interface.

  - Option 2: Transfer the file via SFTP, etc. into the “/home/pi/.venv/” directory.

- Deactivate the Virtual Environment: deactivate

- Set the code to execute on boot by editing the boot script: sudo nano /etc/profile, and add these lines at the very end: 
source .venv/bin/activate
sudo /home/pi/.venv/bin/python3 /home/pi/.venv/tally_light_multi-5Nov25.py
(echo commands can also be added, if desired: echo “Tally Light waiting 30 sec to ensure Wifi is connected.”)

- For advanced users:

  - For some Wifi connections, it takes longer to connect, so adding a sleep 30 to the /etc/profile script can allow that to happen.

  - Additionally, it may be desirable to update the boot script to wait a few loops for a Wifi connection – this would reduce the number of reboots.

  - I added a sleep 30 and sudo reboot at the end of the boot script to force the RPi to reboot if the Python script ever errors out.  This allows the Tally light to always resume it’s “normal” state if something goes wrong (I’ve noticed that OBS crashes can leave the Python script in odd states or even cause it to crash)

For existing Tally Lights, update the Raspbian OS (and Python) to the latest versions can be attempted, but it is difficult and risky – re-imaging the uSD card from scratch using the instructions above is a much better option.  However, here are some steps to try:

- Update the system:  sudo apt update and sudo apt upgrade

- As of Nov25, it seems like the Raspbian repo’s only include Python 3.7 and don’t point to the latest Raspberry Pi OS.  To get around this, we can edit the sources.list to search for a newer version of Raspberry Pi OS:

  - Backup the previous sources.list and raspi.list:

    - sudo cp /etc/apt/sources.list /etc/apt/sources.list.bak

    - sudo cp /etc/apt/sources.list.d/raspi.list /etc/apt/sources.list.d/raspi.list.bak

  - Edit the “sources.list” AND “raspi.list” files to point to the latest version of Raspberry Pi OS (trixie, here):

    - To determine the latest compatible version of Raspberry Pi OS (formerly Raspbian), visit: https://www.raspberrypi.com/software/operating-systems/.  These devices are built on a Raspberry Pi Zero W (the original v1).

    - Open the file with a text editor: sudo nano /etc/apt/sources.list

    - In nano, edit the line to show the name of the latest version (bookworm, in this example):

      - Old: deb http://raspbian.raspberrypi.org/raspbian/ buster main contrib non-free rpi

      - New: deb http://raspbian.raspberrypi.org/raspbian/ bookworm main contrib non-free rpi

    - Repeat for “raspi.list” file: sudo nano /etc/apt/sources.list.d/raspi.list

- Update the Operating system:  sudo apt update and sudo apt upgrade

- Install the new Python 3 web sockets library: sudo pip3 install obsws-python

- sudo apt install pipx

- sudo apt install rpigpio